% Objective: Display raw and averaged spiking data

% What you need: Aligned spiketimes per trial

% Example data: Cortical multi-unit responses to repeated presentation of pure tone

Felix Schneider

Auditory Cognition Group: https://www.auditorycognition.org/

Biosciences Institute, Newcastle University Medical School

02/2020

load('sptimes.mat')

Raster plot

figure('Units','normalized','Position',[0 0 .3 1])

ax = subplot(4,1,1); hold on

% For all trials...

for iTrial = 1:length(sptimes)

spks            = sptimes{iTrial}';         % Get all spikes of respective trial

xspikes         = repmat(spks,3,1);         % Replicate array

yspikes      	= nan(size(xspikes));       % NaN array

if ~isempty(yspikes)

yspikes(1,:) = iTrial-1;                % Y-offset for raster plot

yspikes(2,:) = iTrial;

end

plot(xspikes, yspikes, 'Color', 'k')

end

ax.XLim             = [-.2 .3];

ax.YLim             = [0 length(sptimes)];

ax.XTick            = [0 .2];

ax.XLabel.String  	= 'Time [s]';

ax.YLabel.String  	= 'Trials';

Peristimulus time histogram (PSTH)

all = [];

for iTrial = 1:length(sptimes)

all             = [all; sptimes{iTrial}];               % Concatenate spikes of all trials

end

ax                  = subplot(4,1,2);

nbins               = 100;

h                   = histogram(all,nbins);

h.FaceColor         = 'k';

mVal                = max(h.Values)+round(max(h.Values)*.1);

ax.XLim             = [-.2 .3];

ax.YLim             = [0 mVal];

ax.XTick            = [0 .2];

ax.XLabel.String  	= 'Time [s]';

ax.YLabel.String  	= 'Spikes/Bin';

slength             = 500;                                  % Length of signal trace [ms]

bdur                = slength/nbins;                        % Bin duration in [ms]

nobins              = 1000/bdur;                            % No of bins/sec

for iLab = 1:length(ax.YTickLabel)

lab             = str2num(ax.YTickLabel{iLab});

conv            = (lab / length(sptimes)) * nobins; 	% Convert to [Hz]: avg spike count * bins/sec

newlabel{iLab}  = num2str(round(conv));                 % Change YLabel

end

ax.YTickLabel       = newlabel;

ax.YLabel.String  	= 'Firing Rate [Hz]';

Spike density function

tstep     	= .001;                     % Resolution for SDF [s]

sigma   	= .005;                     % Width of gaussian/window [s]

time     	= tstep-.2:tstep:.3;        % Time vector

for iTrial = 1:length(sptimes)

spks    = [];

gauss   = [];

spks   	= sptimes{iTrial}';          % Get all spikes of respective trial

if isempty(spks)

out	= zeros(1,length(time));    % Add zero vector if no spikes

else

% For every spike

for iSpk = 1:length(spks)

% Center gaussian at spike time

mu              = spks(iSpk);

% Calculate gaussian

p1              = -.5 * ((time - mu)/sigma) .^ 2;

p2              = (sigma * sqrt(2*pi));

gauss(iSpk,:)   = exp(p1) ./ p2;

end

% Sum over all distributions to get spike density function

sdf(iTrial,:)       = sum(gauss,1);

end

end

% Single trial display

ax                  = subplot(4,1,3);

imagesc(sdf)

ax.XLabel.String  	= 'Time [s]';

ax.YLabel.String  	= 'Trials';

ax.XLim             = [1 500];

ax.XTick            = [200 400];

ax.XTickLabel       = {'0', '0.2'};

colormap jet

% Average response

ax = subplot(4,1,4);

plot(mean(sdf), 'Color', 'k', 'LineWidth', 1.5)

mVal = max(mean(sdf)) + round(max(mean(sdf))*.1);

ax.XLim             = [0 500];

ax.YLim             = [0 mVal];

ax.XTick            = [200 400];

ax.XTickLabel       = {'0', '0.2'};

ax.XLabel.String  	= 'Time [s]';

ax.YLabel.String  	= 'Firing Rate [Hz]';